LITERATURE SURVEY

# 1.The notion of a genetic algorithm combined with the Support Vector Machine is presented in this work (SVM). Billing data collected by the agency was first filtered based on criteria such as ignoring customers who have not used their account for 25 months. The load profiling process is then completed. Following that, feature extraction and data normalization were completed. The data was then classified using SVM and separated into four groups. Class 1 clients are extremely likely to be fraudulent, whereas Class 4 customers 1 Authorized licensed use limited to: Zhejiang University. Downloaded on April 06,2024 at 08:57:58 UTC from IEEE Xplore. Restrictions apply. are unlikely to be fraudulent. Genetic algorithm (GA) optimization is then used to minimize the hyperparameters of the SVM to one chromosome, thus identifying cheating with minimal effort. Still, SVM is not a very accurate mechanism for classification, so this approach has accuracy issues. GA reduces overhead, but accuracy remains low.

# 2.This suggested system covers the notion of monitoring power use every half-hour. It requires smart power meters capable of wirelessly transmitting client consumption data to electricity authorities. The information gathered would be utilized for client load profiling. This research uses ideas such as fuzzy logic and intelligent systems. One month of data is used to get the results. First, we collected data from smart meters. Data were then preprocessed for correlation with load profiles. With the help of these load profiles, consumption anomalies were discovered and fraud detected by classifying customers into five categories using fuzzy logic, but such a process requires infrastructure and requirements. results may not be accurate, as the reference month for testing may be a holiday month in which consumption exceeds normal consumption.

# 3.Describes approaches for detecting electricity theft and applying fuzzy logic to enhance power quality. It compares the total load provided by the management power station with the total load consumed by users and uses fault signals to detect power theft. The load (energy) and voltage are sent to a fuzzy logic controller, which provides the associated output voltage variation to enhance power disposition and minimize power theft. The replication is run in MATLAB and the simulation results are displayed. This helps predict the effective use of intelligent control in electrical models. Therefore, using creative authority in the electric model can significantly improve energy efficiency while securing various nonlegal measures. Power theft monitoring becomes a peer of the country's economy to meet power generation requirements. As a result, the development of automatic intelligence in energy systems becomes important.

# 4. This paper describes that the power line communication hypothesis is used for power theft detection. As the line is packed or emptied, a high-frequency signal is delivered into the transmission networks and alters amplitude and frequency. When illegal connections are made between the poles, the amplification values are altered, illegal connections of electricity are discovered, and authorities take appropriate measures to neutralize such connections, but this approach has been tried and tested. Detecting customer theft; illegal use and infrastructure-based.

# 5. Using an idea of the customer's past power consumption patterns, it provides user load profile information that is used to identify abnormal power flows, creating classes of customers that are then further combined into Identify potential fraudulent customers. The essay uses many concepts, such as Support Vector Machines and Extreme Learning Machines. In the course of the detecting procedure, various steps are taken. The pre-processing procedure begins with consumer usage data. These three procedures make up the processing: data selection, separation, and normalization. The feature selection process, which chooses the pertinent facets of the data automatically, comes next. ELM is then used to categorize the data according to the anomalous usage patterns. To find potential electrical fraud, the classified data is then analyzed and further categorized by SVM. But since we are employing SVM. As SVM is less accurate at categorizing data, there is a chance that fraud will not be successfully detected, which reduces the accuracy of detection.

# 6. Smart meters with temperature-based energy meters for predicting electricity theft. It is a model based on the progress of technical loss evaluation based on continuous resistance. They used the model and advanced the resistance estimation by including the temperature variable. Since the variables depend on the materials that make up the transmission line, the theft may be predicted by computing the variables and estimating the circuit. If the theft rate exceeds 4%, the model is suitable for prediction. This model uses the electricity use patterns of users by collecting data from the meter every 30 minutes and taking the temperature profile as input. While this method is effective at detecting rogue network connections, it is significantly more complex and requires more infrastructure. Since circuit calculation is required, an approximation is also required.